#include "esp_camera.h"

#include <WiFi.h>

#include <WebServer.h>

/* ================= CAMERA PINS (AI THINKER) ================= */

#define PWDN_GPIO_NUM 32

#define RESET_GPIO_NUM -1

#define XCLK_GPIO_NUM 0

#define SIOD_GPIO_NUM 26

#define SIOC_GPIO_NUM 27

#define Y9_GPIO_NUM 35

#define Y8_GPIO_NUM 34

#define Y7_GPIO_NUM 39

#define Y6_GPIO_NUM 36

#define Y5_GPIO_NUM 21

#define Y4_GPIO_NUM 19

#define Y3_GPIO_NUM 18

#define Y2_GPIO_NUM 5

#define VSYNC_GPIO_NUM 25

#define HREF_GPIO_NUM 23

#define PCLK_GPIO_NUM 22

/* ================= MOTOR PINS ================= */

#define ENA 12

#define IN1 13

#define IN2 15

#define ENB 2

#define IN3 14

#define IN4 4 // DO NOT use 4 here (flash LED uses 4)

/* ================= BUILT IN FLASH LED ================= */

#define FLASH_LED 4

/* ================= WIFI ================= */

const char* ssid = "ESP32-CAR";

const char* password = "12345678";

WebServer server(80);

/* ================= PWM ================= */

#define PWM_FREQ 5000

#define PWM_RES 8

int speedValue = 150;

/* ================= HTML ================= */

String webpage = R"====(

<!DOCTYPE html>

<html>

<head>

<meta name="viewport" content="width=device-width, initial-scale=1">

<style>

button {width:70px;height:70px;font-size:30px;margin:5px;}

</style>

</head>

<body align=center>

<h2>ESP32 CAM CAR</h2>

<img src="/stream" width="320"><br><br>

<button onclick="fetch('/forward')">⬆</button><br>

<button onclick="fetch('/left')">⬅</button>

<button onclick="fetch('/stop')"></button>

<button onclick="fetch('/right')">➡</button><br>

<button onclick="fetch('/backward')">⬇</button>

<br><br>

Speed:<br>

<input type="range" min="0" max="255" value="150"

oninput="fetch('/speed?value='+this.value)">

<br><br>

LED Brightness:<br>

<input type="range" min="0" max="255" value="0"

oninput="fetch('/light?value='+this.value)">

</body>

</html>

)====";

/* ================= MOTOR CONTROL ================= */

void stopCar(){

digitalWrite(IN1,LOW);

digitalWrite(IN2,LOW);

digitalWrite(IN3,LOW);

digitalWrite(IN4,LOW);

}

void forward(){

digitalWrite(IN1,HIGH);

digitalWrite(IN2,LOW);

digitalWrite(IN3,HIGH);

digitalWrite(IN4,LOW);

}

void backward(){

digitalWrite(IN1,LOW);

digitalWrite(IN2,HIGH);

digitalWrite(IN3,LOW);

digitalWrite(IN4,HIGH);

}

void left(){

digitalWrite(IN1,LOW);

digitalWrite(IN2,HIGH);

digitalWrite(IN3,HIGH);

digitalWrite(IN4,LOW);

}

void right(){

digitalWrite(IN1,HIGH);

digitalWrite(IN2,LOW);

digitalWrite(IN3,LOW);

digitalWrite(IN4,HIGH);

}

/* ================= CAMERA STREAM ================= */

void startCamera(){

camera_config_t config;

config.ledc_channel = LEDC_CHANNEL_0;

config.ledc_timer = LEDC_TIMER_0;

config.pin_d0 = Y2_GPIO_NUM;

config.pin_d1 = Y3_GPIO_NUM;

config.pin_d2 = Y4_GPIO_NUM;

config.pin_d3 = Y5_GPIO_NUM;

config.pin_d4 = Y6_GPIO_NUM;

config.pin_d5 = Y7_GPIO_NUM;

config.pin_d6 = Y8_GPIO_NUM;

config.pin_d7 = Y9_GPIO_NUM;

config.pin_xclk = XCLK_GPIO_NUM;

config.pin_pclk = PCLK_GPIO_NUM;

config.pin_vsync = VSYNC_GPIO_NUM;

config.pin_href = HREF_GPIO_NUM;

config.pin_sscb_sda = SIOD_GPIO_NUM;

config.pin_sscb_scl = SIOC_GPIO_NUM;

config.pin_pwdn = PWDN_GPIO_NUM;

config.pin_reset = RESET_GPIO_NUM;

config.xclk_freq_hz = 20000000;

config.pixel_format = PIXFORMAT_JPEG;

config.frame_size = FRAMESIZE_QVGA;

config.jpeg_quality = 12;

config.fb_count = 1;

esp_camera_init(&config);

}

void handleStream(){

WiFiClient client = server.client();

String response = "HTTP/1.1 200 OK\r\n";

response += "Content-Type: multipart/x-mixed-replace; boundary=frame\r\n\r\n";

server.sendContent(response);

while(client.connected()){

camera_fb_t * fb = esp_camera_fb_get();

if(!fb) return;

server.sendContent("--frame\r\n");

server.sendContent("Content-Type: image/jpeg\r\n\r\n");

server.sendContent((char*)fb->buf, fb->len);

server.sendContent("\r\n");

esp_camera_fb_return(fb);

}

}

/* ================= SETUP ================= */

void setup(){

Serial.begin(115200);

pinMode(IN1,OUTPUT);

pinMode(IN2,OUTPUT);

pinMode(IN3,OUTPUT);

pinMode(IN4,OUTPUT);

// PWM for motors

ledcAttach(ENA, PWM_FREQ, PWM_RES);

ledcAttach(ENB, PWM_FREQ, PWM_RES);

ledcWrite(ENA, speedValue);

ledcWrite(ENB, speedValue);

// PWM for Flash LED

ledcAttach(FLASH_LED, PWM_FREQ, PWM_RES);

ledcWrite(FLASH_LED, 0);

startCamera();

WiFi.softAP(ssid,password);

Serial.println(WiFi.softAPIP());

server.on("/", [](){ server.send(200,"text/html",webpage); });

server.on("/stream", handleStream);

server.on("/forward", forward);

server.on("/backward", backward);

server.on("/left", left);

server.on("/right", right);

server.on("/stop", stopCar);

server.on("/speed", [](){

speedValue = server.arg("value").toInt();

ledcWrite(ENA, speedValue);

ledcWrite(ENB, speedValue);

server.send(200,"text/plain","OK");

});

server.on("/light", [](){

int brightness = server.arg("value").toInt();

ledcWrite(FLASH_LED, brightness);

server.send(200,"text/plain","OK");

});

server.begin();

}

/* ================= LOOP ================= */

void loop(){

server.handleClient();

}